| Service Memo |  | 2020-002 |
| --- | --- | --- |
| Machine: | Schaerer Coffee Soul |  |
| Titel: | How to update software from version 4.0.5 to 4.12.3 |  |

Dear All,

Hereby an extra service memo for the Schaerer Coffee Soul with respect to how to update software from version 4.0.5 directly to 4.12.3. This you will do if you change the proportional valve into a fixed 2 way valve.

So if you have software version 4.0.5 and want to change to a fixed valve instead of the proportional valve then you need to do this in the following order:

- Switch off he machine

- Change the proportional valve into the fixed 2 way valve

- Switch on the machine

- Update the machine to software to version 4.12.3

- Do a hardware detection

- Do a master reset

- Do a commissioning

- Machine is now up to date and working correctly.

If you are not doing it in this order you can get strange behaviours of the machine and things are switch on in the setting even when they are not as hardware in the machine. Only the master reset will helps in this case.