Service memo

15 June 2022

Number:		JDE 2022-03

Dispenser(s):	SOUL Barista

Title:			Configuration information SOUL BARISTA

With this service memo I want to give you some information with respect to the SOUL BARISTA configuration It is a little bit different than the SOUL 10-12 configuration.

The 2 things which are different are:

- The SOUL BARISTA has an inline sensor to detect milk empty. This is done because by using bag in box milk the regular empty sensor is not working. Sensor is located just above the best foam mixer.

- The SOUL BARISTA has additional water instead of the by-pass water (brewer accelerator) as in the SOUL 10-12. The difference is that with the additional water there is a separate water outlet and with by-pass water it will merge with the coffee water and end up at the outlet.

For detailed explanation please check the Service Module Coffee Soul HARDWARE_EN V14.00 module which you can find on JDE support.