Service memo

24 January 2023

Number:		JDE 2023-01

Dispenser(s):	Schaerer Coffee Art Best Foam

Title:	ECO RINSE explanation

With this service memo I want to inform you that with software version 2.13 for the Schaerer Coffee Art Best Foam Schaerer introduced the ECO RINSE.

The new "Eco rinse" function reduces milk consumption during repetitive rinsing of the milk lines.

With an active eco rinse, the lines are left with water at the end of the rinse. Before a beverage is dispensed, the lines are filled with milk.

We also expect that the blockage of the drain will also be less as a result, but this has not been investigated further, so it is an assumption.

In service information 2022-06_(SCA_P_BF)_(EN)_Software_V2.13 is explained where you can find the settings in the software for ECO RINSE to switch on.

Service mode> General parameters > Eco Rinse