Service memo

27 January 2023

Number:		JDE 2023-02

Dispenser(s):	Schaerer SCS/SOUL 10-12-Barista

Title:	Back up restore beverage/ingredient settings/menu card explanation

With this service memo I would like to indicate once again how you can best restore a backup of a machine. In case of a master reset, of course, upload the latest software and check whether everything is correct set in the machine. Perform a hardware detection and then perform the calibrations.

Below is how you can best restore the recipes, ingredients and menu from a backup:

You can use your own backup or use the backup that you can find on JDE support.

Proceed as follows:

• Go to the technician menu.
• Insert the USB stick with the backup into the USB slot
• Navigate to the Service menu
• Select Import data.
• Select Restore Settings
• Slide the top Select All out so that all selection fields are greyed out
• Only select Product recipes / Ingredients / Menu card
• Give an Enter (arrow) at the top of the screen
• Select the USB stick and the correct file
• Push

The machine will restart and the screen should now be filled with the possible product choices.