Service memo

12-12-2024

Number:		JDE 2024-10 update 2

Dispenser(s):	Schaerer SKYE

Title:	Schaerer Skye new valve and twin milk available

From week 27 there is a ‘new’ valve installed in the Schaerer Coffee Skye.

Due to this new valve it is also possible to have twin milk on the SKYE, only with 10 liter fridge.

For single milk valve 16 Switch over valve (55125958 (100394), 3mm) in the picture below is replaced by 55126719 (101060).

Skye twin milk:

After week 27 it is also possible to have twin milk in the SKYE only with 10 Liter fridge.

If you want to have this you have to rebuild like described below:

You have to add an extra valve 55126719 (so now valve 1 (V18) and 2 (V20) are for twin milk) to the system and do a hardware detection.

Hardware looks like:

Wire from V16 (valve before week 27) will be connected to V18 and V20 has to connected by the not used wire, see below:

Extra needed beside the valves for the Fridge:

1x 55124937	Milk container bracket TM		1x 55126720	Bayonet lock 6 pps socket
1x 55116941	Riser pipe 260 milk container		2x 55118040	Plug-in connection L 4-6
1x 55119848	Hose adapter Plug & Clean 2K		2x 55113991	Hose 2.5/4 PFA
2x 55124936	Milk container 5l
1x 55026144	Hose 5/8 SILICONE BLUE
1x 55125917	Hose 4/8 SILICONE ORANGE