Maintenance Protocol

Version:	1.1 (10/2020)

Machine Type:		     		Serial Number:

Customer:

Technician Name:

Service Report No:	     		Date:

| Functional test |  | Remark on error |
| --- | --- | --- |
| Check device for function: Tap all types of water, Check alarm list |  |  |
| Measurement of electrical safety according to DGUV v3 |  |  |
| Optical inspection for leaks |  |  |
| Cleaning |  |  |
| Carry out dry cleaning (according to the instructions in Dozuki) |  |  |
| Cleaning of the device inside and outside |  |  |
| Cleaning and descaling the taps and the drip tray |  |  |
| Cleaning the cooling fins |  |  |
| Checking the device |  |  |
| Check for external damage |  |  |
| Check for completeness of the system |  |  |
| Checking the sewage connection |  |  |
| Checking the water connection |  |  |
| Water tightness test |  |  |
| Testing the water flow |  |  |
| Testing the water pressure reducer |  |  |
| Testing the C02 connection |  |  |
| Test C02 pressure reducer incl. pressure relief valve |  |  |
| Testing of the C02 tightness |  |  |
| Testing of the hose cross sections |  |  |
| Testing the carbonator pump |  |  |
| Only Vivreau Fill 200: Check Kalibration Nozzle 1/2 |  |  |
| Only Vivreau Fill 200: Check ice bank level |  |  |
| Exchange of maintenance relevant components |  |  |
| Replacement of water inlet filter (insert pre-rinsed in the course of chemical cleaning) |  |  |
| Only Option Hygienic Plus: change X3 Filter |  |  |
| Exchange HMI battery (every 4 years, only Vivreau Top/Extra/Fill) |  |  |
| Final activities |  |  |
| Measurement of electrical safety according to DGUV v3 |  |  |
| Check device for function: Tap all types of water, Check alarm list |  |  |
| Entry in operating log / cleaning proof |  |  |
| Maintenance sticker attached |  |  |
| Service report signed by the customer |  |  |